- 各国电源线标准及所使用插头规格

1. 中国电源线标准及所用插头规格

227IEC42(RVB)2×0.5mm2   6A250V
227IEC42(RVB)2×0.75mm2  6A250V
227IEC52(RVV)2×0.5mm2   6A250V
227IEC52(RVV)2×0.75mm2  6A250V

------------------------------------------------------------------------------

227IEC52(RVV) 2×0.75mm2 6A250V
227IEC53(RVV) 2×0.75mm2 6A250V
227IEC53(RVV) 2×1.0mm2  10A250V
--------------------------------------------------------------------------

227IEC42(RVB)2×0.5mm2   6A250V
227IEC42(RVB)2×0.75mm2  6A250V
227IEC52(RVV)2×0.5mm2   6A250V
227IEC52(RVV)2×0.75mm2  6A250V
----------------------------------------------------------------------------

227IEC52（RVV）3×0.5mm2   6A250V
227IEC52（RVV）3×0.75mm2  6A250V
227IEC53（RVV）3×0.75mm2  6A250V
227IEC53（RVV）3×1.0mm2   10A250V
-----------------------------------------------------------------------

227IEC52（RVV）3×0.5mm2   6A250V
227IEC52（RVV）3×0.75mm2  6A250V
227IEC53（RVV）3×0.75mm2  6A250V
227IEC53（RVV）3×1.0mm2   10A250V
-----------------------------------------------------------------------

227IEC52（RVV）3×0.5mm2   6A250V
227IEC52（RVV）3×0.75mm2  6A250V
227IEC53（RVV）3×0.75mm2  6A250V
227IEC53（RVV）3×1.0mm2   10A250V
RX 300/300V    3×0.75mm2  6A250V
RX 300/300V    3×1.0mm2   10A250V
------------------------------------------------------------------------

2. 欧洲电源线标准及所对应插头规格

-------------------------------------------------------------------------

H03VVH2 -F  2X0.5mm2   10/16A250V
H03VVH2 -F  2X0.75mm2  10/16A250V
H03VV -F 2X0.5mm2   10/16A250V
H03VV -F  2X0.75mm2  10/16A250V
H05VVH2 -F  2X0.75mm2  10/16A250V
H05VVH2 -F  2X1.0mm2  10/16A250V
--------------------------------------------------------------------------

H05VV-F  2X0.75mm2 10/16A250V
H05VV-F  2X1.0mm2 10/16A250V
H03VVH2-F  2X0.5mm2  2.5A250V
H03VVH2 -F  2X0.75mm2 2.5A250V
H03VV -F  2X0.5mm2  2.5A250V
H03VV -F  2X0.75mm2 2.5A250V
H05VVH2 -F  2X0.75mm2 10A250V
H05VVH2 -F  2X1.0mm2 10A250V
H05VV -F  2X0.75mm2 10A250V
H05VV -F  2X1.0mm2 10A250V

--------------------------------------------------------------------

H03VV -F 3G0.5mm2  10/16A250V
H03VV -F 3G0.75mm2 10/16A250V
H05VV -F 3G0.75mm2 10/16A250V
H05VV -F 3G1.0mm2 10/16A250V
H05VV -F 3G1.5mm2 10/16A250V

------------------------------------------------------------------------

H03VV -F 3G0.5mm2  10/16A250V
H03VV -F 3G0.75mm2 10/16A250V
H05VV -F 3G0.75mm2 10/16A250V
H05VV -F 3G1.0mm2 10/16A250V
H05VV -F 3G1.5mm2 10/16A250V

--------------------------------------------------------------------------

H03VV -F  3G0.5mm2  10A250V
H03VV -F  3G0.75mm2 10A250V
H05VV -F  3G0.75mm2 10A250V
H05VV -F  3G1.0mm2 10A250V
-------------------------------------------------------------------------

H03VV -F  3G0.5mm2  10A250V
H03VV -F  3G0.75mm2 10A250V
H05VV -F  3G0.75mm2 10A250V
H05VV -F  3G1.0mm2 10A250V

-------------------------------------------------------------------------

227IEC52(RVV) 3X0.75mm2 10A250V
227IEC53(RVV) 3X0.75mm2 10A250V
227IEC53(RVV) 3X1.0mm2 10A250V
H03VV -F 3G0.75mm2 6A250V 
H05VV -F 3G0.75mm2 10A250V
H05VV -F 3G1.0mm2 10A250V

-------------------------------------------------------------------------

227IEC52(RVV) 3X0.75mm2 10A250V
227IEC53(RVV) 3X0.75mm2 10A250V
227IEC53(RVV) 3X1.0mm2 10A250V
H03VV -F 3G0.75mm2 6A250V 
H05VV -F 3G0.75mm2 10A250V
H05VV -F 3G1.0mm2 10A250V
-------------------------------------------------------------------------

3.偶大利亚电源线标准及所用插头规格

-------------------------------------------------------------------------

H05VVH2 -F 2X0.75mm2 7.5A250V
H05VV -F 2X0.75mm2 7.5A250V
H05VV -F 2X1.0mm2 10A250V

-------------------------------------------------------------------------

H05VVH2 -F 2X0.75mm2 7.5A250V
H05VV -F 2X0.75mm2 7.5A250V
H05VV -F 2X1.0mm2 10A250V

-------------------------------------------------------------------------

H05VV -F 3G0.75mm2 7.5A250V
H05VV -F 3G1.0mm2 10A250V
H05VV -F 3G1.5mm2 15A250V
-------------------------------------------------------------------------

4.美国电源线标准及所有插头规格

-------------------------------------------------------------------------

SPT-1 18AWG/2C
SPT-2 18AWG/2C
SPT-2 16AWG/2C
SPT-3 18AWG/2C
SPT-3 16AWG/2C
SVT 18AWG/2C
SVT 17AWG/2C
NISPT-1 18AWG/2C
NISPT-2 18AWG/2C

-------------------------------------------------------------------------

SVT 18AWG/2C
SVT 17AWG/2C
ST 18AWG/2C
ST 16AWG/2C
SJT 18AWG/2C
SJT 16AWG/2C
NISPT-1 18AWG/2C
NISPT-2 18AWG/2C

-------------------------------------------------------------------------

SVT 18AWG/2C
SVT 17AWG/2C
SVT 18AWG/2C(齿形)
SVT 17AWG/2C(齿形)
ST 18AWG/2C
ST 16AWG/2C

-------------------------------------------------------------------------

SPT-1 18AWG/3C
SPT-2 18AWG-16AWG/3C
SPT-3 18AWG-16AWG/3C
SVT 18AWG- 17AWG/3C
SVT 18AWG-17AWG/3C(齿形)
ST 18AWG-16AWG/3C
SJT  18AWG-16AWG/3C

-------------------------------------------------------------------------

SVT 18AWG/3C
SVT 17AWG/3C
SVT 18AWG/3C(齿形)
SVT 17AWG/3C(齿形)
ST 18AWG/3C
ST 16AWG/3C
SJT  16AWG/3C
SJT  18AWG/3C

-------------------------------------------------------------------------

5.日本电源线标准及所对应插头规格

-------------------------------------------------------------------------

VFF 2X0.75mm2 7A125V
VCTF 2X0.75mm2 7A125V
VCTFK 2X0.75mm2 7A125V

-------------------------------------------------------------------------

6.以色列电源线标准及所用插头规格

-------------------------------------------------------------------------

H03VV-F 3G0.75mm2
H05VV-F 3G0.75mm2
H05VV-F 3G1.0mm2
H05VV-F 3G1.5mm2

-------------------------------------------------------------------------

7.南非电源线标准及所用插头规格

-------------------------------------------------------------------------

H03VV-F 3G0.75mm2
H05VV-F 3G0.75mm2
H05VV-F 3G1.0mm2
H05VV-F 3G1.5mm2

-------------------------------------------------------------------------

8.英国电源线标准及所用插头规格

-------------------------------------------------------------------------

H03VV-F 3G0.5mm2
H03VV-F 3G0.75mm2
H05VV-F 3G0.75mm2
H05VV-F 3G1.0mm2
H05VV-F 3G1.5mm2

Tags：各国标准 电源线 插头 规格

欧洲标准电源线

| ·; 标准:; DIN VDE0281. ·; 型号:; H03VVH2-F H03VV-F H05VVH2-F H05VV-F. ·; 额定电压:; H03 300/300V. H05 300/500V. ·; 应用:; H03型适用小功率家用电器、电动工具及动力照明等。 H05型适用大功率家用电器电动工具及照明等. ·; 构造. |
| --- |
| H03VVH2-F & H05VVH2-F |  |
| H03VV-F & H05VV-F |  |
| H03VVH2-F & H03VV-F |  |
| 芯数; 面积(mm²); 结构(No/mm); 绝缘厚度(mm); 护套厚度(mm); 完成外径(mm). 2; 0.5; 28/0.15; 0.5; 0.5; 3.3X5.4. 2; 0.5; 28/0.15; 5.4. 2; 0.75; 42/0.15; 3.4X5.6. 2; 0.75; 42/0.15; 5.6. 3; 0.5; 28/0.15; 5.6. 3; 0.75; 42/0.15; 6.1. |
| H05VVH2-F & H05VV-F |  |
| 芯数; 面积(mm²); 结构(No/mm); 绝缘厚度(mm); 护套厚度(mm); 完成外径(mm). 2; 0.75; 42/0.15; 0.6; 0.8; 6.5. 2; 0.75; 42/0.15; 4.1X6.5. 2; 1.0; 32/0.2; 6.7. 2; 1.0; 32/0.2; 4.3X6.9. 2; 1.5; 48/0.2; 0.8; 7.7. 3; 0.75; 42/0.15; 0.6; 6.8. 3; 1.0; 32/0.2; 7.3. 3; 1.5; 48/0.15; 0.8; 0.9; 8.4. |

中国电源线标准

| 导线 |  |  |  |  |
| --- | --- | --- | --- | --- |
| ·; 标准:; GB5023.3-1997. ·; 型号:; 227 IEC 02(RV),227 IEC 08(RV-90). ·; 额定电压:; 227 IEC 02(RV) 450/750V. 227 IEC 08(RV-90) 300/500V. ·; 应用:; 02型适用各种家用电器、仪器仪表连接等. 08型适用各种家用电器高温下连接. |  |  |
| · 构造 |  |  |  |  |
| 227 IEC 02(RV) & 227 IEC 08(RV-90) |  |  |  |  |
| 227 IEC 02(RV) 导体; 绝缘. 面积(mm²); 构造(No./mm); 标称厚度(mm); 完成外径(mm). 1.5; 48/0.2; 0.7; 3.0. 2.5; 80/0.2; 0.8; 3.65. 227 IEC 08(RV-90) 导体; 绝缘. 面积(mm²); 构造(No./mm); 标称厚度(mm); 完成外径(mm). 0.75; 42/0.15; 0.6; 2.6. 1.0; 32/0.2; 0.6; 2.8. 1.5; 48/0.2; 0.7; 3.05. 2.5; 80/0.2; 0.8; 3.7. |  |  |

扁型软线

·; 标准:; GB5023.5-1997. ·; 型号:; 227 IE C42(RVB). ·; 额定电压:; 300/300V. ·; 应用:; 适用于小功率家用电器及照明。.

·构造

227 IEC 42(RVB)

227 IEC 42(RVB)

导体; 绝缘厚度(mm); 完成外径(mm). 芯数; 面积(mm²); 结构(No./mm). 2; 0.5; 28/0.15; 0.8; 2.7×5.4. 2; 0.75; 42/0.15; 0.8; 2.9×5.8.

护套软电线

·; 标准:; GB5023.5-1997. ·; 型号:; 227 IEC 52(RVV),227 IEC 53(RVV). ·; 额定电压:; 227 IEC 52(RVV)300/300V. 227 IEC 53(RVV)300/500V. ·; 应用:; 227 IEC 52(RVV)适用于小功率家用电器、电 动工具以及动力照明等。. 227 IEC 53(RVV)适用于大功率家用电器、电 动工具以及动力照明等。. ·; 构造.

| 227 IEC 52(RVV) & 227 IEC 53(RVV) |  |
| --- | --- |
| 扁型 |  |
| 圆型芯数 |  |
| 227 IEC 52(RVV) |  |
| 芯数; 面积(mm²); 结构(No/mm); 绝缘厚度(mm); 护套厚度(mm); 完成外径(mm). 2; 0.5; 28/0.15; 0.5; 0.6; 5.3. 2; 0.5; 28/0.15; 3.26×5.3. 2; 0.75; 42/0.15; 5.7. 2; 0.75; 42/0.15; 3.4×5.8. 3; 0.5; 28/0.15; 5.6. 3; 0.75; 42/0.15; 6.05. |
| 227 IEC 53(RVV) |  |
| 芯数; 面积(mm²); 结构(No/mm); 绝缘厚度(mm); 护套厚度(mm); 完成外径(mm). 2; 0.75; 42/0.15; 0.6; 0.8; 6.5. 2; 0.75; 42/0.15; 0.6; 4.1×6.5. 2; 1.0; 32/0.2; 0.6; 6.9. 2; 1.5; 48/0.2; 0.7; 7.7. 3; 0.75; 42/0.15; 0.6; 6.9. 3; 1.0; 32/0.2; 0.6; 7.4. 3; 1.5; 48/0.2; 0.7; 0.9; 8.5. |

日本电源线标准

| VFF |  |  |  |
| --- | --- | --- | --- |
| ·; 标准:; JIS C 3306. ·; 型号:; VFF. ·; 额定电压:; 300V. ·; 使用温度:; 75°C. ·; 应用:; 适用于各种家用电器、电动工具及 动力照明。. |  |
| · 构造 |  |  |  |
| 芯数; 面积(mm²); 结构(No/mm); 绝缘厚度(mm); 完成外径(mm). 2; 0.5; 20/0.18; 0.8; 2.5×5.0. 2; 0.75; 30/0.18; 2.7×5.4. 2; 1.25; 50/0.18; 3.1×6.2. |  |

| VCTFK & VCTF |  |  |  |
| --- | --- | --- | --- |
| ·; 标准:; JIS C 3306. ·; 型号:; VCTFK&VCTF. ·; 额定电压:; 300V. ·; 使用温度:; 75°C. ·; 应用:; 适用于各种家用电器、电动工具及 动力照明。. ·; 构造. |  |  |
| VCTFK |  |  |  |
| VCTF |  |  |  |
| VCTFK & VCTF |  |  |  |
| 型号; 芯数; 面积(mm²); 结构(No/mm); 绝缘厚度(mm); 护套厚度(mm); 完成外径(mm). VCTFK; 2; 0.75; 30/0.18; 0.6; 1.0; 4.2×6.5. 2; 1.25; 50/0.18; 4.7x7.4. VCTF; 2; 0.75; 30/0.18; 6.6. 2; 1.25; 50/0.18; 7.4. 3; 0.75; 30/0.18; 7.0. 3; 1.25; 50/0.18; 8.5. |  |  |

美国电源线标准

|  | C-UL |  |
| --- | --- | --- |
|  | ·; 标准:; UL62. ·; 型号:; SPT-1，SPT-2，SPT-3，NISPT-2， SVT，ST，SJT. ·; 额定电压:; 300V，600V（ST）. ·; 使用温度:; 60°C/105°C. ·; 应用:; 适用于各种家用电器、电动工具及 动力照明。. |
|  | · 构造 |  |
|  | SPT-1 & SPT-2 & SPT-3 |  |
|  | 型号; 芯数; 线规; 面积(mm²); 结构(No/mm); 护套厚度(mm); 完成外径(mm). SPT-1; 2; 18; 0.823; 41/0.16; 0.76; 2.7X5.4. 3; 18; 0.823; 41/0.16; 0.76; 2.7X7.5. SPT-2; 2; 18; 0.823; 41/0.16; 1.14; 3.5X6.9. 2; 16; 1.31; 2.6/0.253; 3.7X7.5. 3; 18; 0.823; 41/0.16; 3.6X8.6. 3; 16; 1.31; 26/0.253; 3.9X9.6. SPT-3; 2; 18; 0.823; 41/0.16; 1.52; 4.4X7.8. 2; 16; 1.31; 26/0.253; 4.5X9.1. 3; 18; 0.823; 41/0.16; 4.3X9.8. 3; 16; 1.31; 26/0.253; 4.6X11.8. |

NISPT-1 & NISPT-2

型号; 芯数; 线规; 面积(mm²); 结构(No/mm); 绝缘厚度(mm); 护套厚度(mm); 完成外径(mm). NISPT-1; 2; 18; 0.823; 41/0.16; 0.38; 0.38; 3.1x6.0. NISPT-2; 2; 18; 0.823; 41/0.16; 0.76; 0.38; 4.0x7.6.

| SVT & ST & SJT |  |
| --- | --- |
| 型号; 芯数; 线规; 面积(mm²); 结构(No/mm); 绝缘厚度(mm); 护套厚度(mm); 完成外径(mm). SVT; 2; 18; 0.823; 41/0.16; 0.38; 0.76; 5.9±0.1. 3; 18; 0.823; 41/0.16; 6.3±0.1. 2; 17; 1.04; 52/0.16; 6.4±0.1. 3; 17; 1.04; 52/0.16; 6.8±0.1. ST; 2; 18; 0.823; 41/0.16; 0.76; 1.52; 8.5±0.15. 3; 18; 0.823; 41/0.16; 8.9±0.15. 2; 16; 1.31; 26/0.253; 9.3±0.15. 3; 16; 1.31; 26/0.253; 9.8±0.15. SJT; 2; 18; 0.823; 41/0.16; 0.76; 0.76; 7.4±0.15. 2; 16; 1.31; 26/0.253; 7.8±0.15. 2; 14; 2.08; 41/0.253; 8.6±0.15. 3; 18; 0.823; 41/0.16; 7.8±0.15. 3; 16; 1.31; 26/0.253; 8.4±0.15. 3; 14; 2.08; 41/0.253; 9.3±0.15. |

澳洲电源线标准

| SAA |  |
| --- | --- |
| ·; 标准:; SAA AS 3191-1994. ·; 型号:; H03VVH2-F H03VV-F H05VVH2-F H05VV-F. ·; 额定电压:; H03 300/300V. H05 300/500V. ·; 应用:; H03型适用小功率家用电器、电动工具及动力照明等。 H05型适用大功率家用电器电动工具及照明等. ·; 构造. |
| H03VVH2-F & H05VVH2-F |  |
| H03VV-F & H05VV-F |  |
| H03VVH2-F & H03VV-F |  |
| 芯数; 面积(Sqmm); 结构(No/mm); 绝缘厚度(mm); 护套厚度(mm); 完成外径(mm). 2; 0.5; 28/0.15; 0.5; 0.5; 3.3X5.4. 2; 0.5; 28/0.15; 5.4. 2; 0.75; 42/0.15; 3.7X5.8. 2; 0.75; 42/0.15; 5.6. 3; 0.5; 28/0.15; 5.6. 3; 0.75; 42/0.15; 6.1. |
| H05VVH2-F & H05VV-F |  |
| 芯数; 面积(Sqmm); 结构(No/mm); 绝缘厚度(mm); 护套厚度(mm); 完成外径(mm). 2; 0.75; 42/0.15; 0.6; 0.8; 6.5. 2; 0.75; 42/0.15; 4.1X6.5. 2; 1.0; 32/0.2; 6.9. 2; 1.5; 48/0.2; 0.8; 7.7. 3; 0.75; 42/0.15; 0.6; 6.8. 3; 1.0; 32/0.2; 7.3. 3; 1.5; 48/0.15; 0.8; 0.9; 8.4. |